# SparseFlow Simulator

A Cycle-Accurate RISC-V Processor with Sparse Matrix Extensions

Documentation & Source Code Reference

Language: C++17

Submitted by:

| Name | Qalam ID |
| --- | --- |
| Rimsha Nosheen | 471832 |
| Sameer Ahmed | 467192 |
| Qura-Tul-Ain Azhar | 460239 |

# Table of Contents

1. Architecture Overview

2. Directory Structure

3. Core Module (src/core/)

3.1 SparseFlowCore.h/.cpp

3.2 PipelineRegisters.h

4. ISA Module (src/isa/)

4.1 Instruction.h

4.2 ISA.h

5. Hardware Module (src/hardware/)

5.1 Memory.h/.cpp

5.2 RegisterFile.h/.cpp

5.3 Cache.h/.cpp

5.4 BranchPredictor.h/.cpp

6. Utilities Module (src/utils/)

6.1 Loader.h/.cpp

6.2 Stats.h/.cpp

6.3 TraceLogger.h/.cpp

7. Main Entry Point

8. Class Diagrams

9. Data Flow

10. API Reference

# 1. Architecture Overview

The SparseFlow Simulator is a cycle-accurate processor simulator implementing a 5-stage pipeline based on the RISC-V instruction set with custom extensions for sparse matrix operations.

## 1.1 System Components

| Component | Description |
| --- | --- |
| SparseFlowCore | Main pipeline controller managing the 5-stage execution |
| Memory | Instruction and data memory with cache simulation |
| RegisterFile | 32 scalar registers + 8 vector registers |
| BranchPredictor | 1-bit dynamic branch predictor |
| Cache | Direct-mapped L1 cache with configurable size |
| Loader | Assembly file parser with label resolution |
| Stats | Performance metrics collector and reporter |
| TraceLogger | CSV-based pipeline trace generator |

## 1.2 Pipeline Stages

| Stage | Function |
| --- | --- |
| IF (Fetch) | Fetches instruction from instruction memory using PC. Applies branch prediction. |
| ID (Decode) | Decodes opcode, reads register file. Detects load-use hazards and inserts stalls. |
| EX (Execute) | Performs ALU operations. Implements data forwarding. Resolves branches. |
| MEM (Memory) | Accesses data memory for load/store. Simulates cache latency. |
| WB (Writeback) | Writes results back to register file. Updates forwarding shadow register. |

# 2. Directory Structure

src/
├── main.cpp                    # Program entry point
├── core/                       # Pipeline core logic
│   ├── SparseFlowCore.h        # Core class declaration
│   ├── SparseFlowCore.cpp      # Pipeline implementation
│   └── PipelineRegisters.h     # Pipeline register structs
├── isa/                        # Instruction Set Architecture
│   ├── Instruction.h           # Opcode enum and Instruction struct
│   └── ISA.h                   # Register ABI mappings
├── hardware/                   # Hardware component models
│   ├── Memory.h/.cpp           # RAM and instruction memory
│   ├── RegisterFile.h/.cpp     # Scalar and vector registers
│   ├── Cache.h/.cpp            # Direct-mapped cache
│   └── BranchPredictor.h/.cpp  # 1-bit dynamic predictor
└── utils/                      # Utility classes
    ├── Loader.h/.cpp           # Assembly file parser
    ├── Stats.h/.cpp            # Performance statistics
    └── TraceLogger.h/.cpp      # Pipeline trace output

# 3. Core Module (src/core/)

The core module contains the main pipeline controller and pipeline register definitions.

## 3.1 SparseFlowCore.h

The main class that orchestrates the 5-stage pipeline simulation.

### Class Declaration

class SparseFlowCore {
private:
    int pc;                          // Program Counter
    long long cycle_count;           // Total cycles executed
    bool halted;                     // Simulation stop flag

    // Hardware Units
    RegisterFile reg_file;           // 32 scalar + 8 vector registers
    Memory memory;                   // Instruction and data memory
    BranchPredictor branch_predictor;// 1-bit dynamic predictor
    Stats stats;                     // Performance metrics
    TraceLogger trace_logger;        // CSV trace output

    // Pipeline Registers
    IF_ID_Reg  if_id;               // Fetch → Decode
    ID_EX_Reg  id_ex;               // Decode → Execute
    EX_MEM_Reg ex_mem;              // Execute → Memory
    MEM_WB_Reg mem_wb;              // Memory → Writeback
    MEM_WB_Reg wb_shadow;           // Shadow register for WB forwarding

    // Pipeline Control
    int zero_skips;                 // LNZ zero-skip counter
    bool stall_pipeline;            // Stall flag for hazards
    int mem_stall_cycles;           // Memory latency counter
    bool single_cycle_mode;         // Single-cycle execution flag

    // Pipeline Stage Methods
    void stageFetch();
    void stageDecode();
    void stageExecute();
    void stageMemory();
    void stageWriteback();

public:
    SparseFlowCore();
    void loadProgram(const std::vector<Instruction>& prog);
    void writeDataMemory(int addr, int val);
    void run();
    void setSingleCycleMode(bool enable);

private:
    bool isPipelineEmpty() const;
};

### Key Methods

| Method | Description |
| --- | --- |
| SparseFlowCore() | Constructor. Initializes PC to 0, cycle count to 0, halted to false. |
| loadProgram(prog) | Loads a vector of Instructions into instruction memory. |
| writeDataMemory(addr, val) | Pre-initializes data memory before simulation. |
| run() | Main simulation loop. Executes stages in reverse order until HALT. |
| setSingleCycleMode(enable) | Enables single-cycle mode (drains pipeline after each instruction). |
| stageFetch() | Fetches instruction at PC. Applies branch prediction for BEQ/JMP. |
| stageDecode() | Decodes instruction, reads registers. Detects load-use hazards. |
| stageExecute() | Performs ALU ops. Implements forwarding from MEM and WB stages. |
| stageMemory() | Accesses data memory for LW/SW. Handles cache hit/miss latency. |
| stageWriteback() | Writes result to register file. Updates wb_shadow for forwarding. |

## 3.2 PipelineRegisters.h

Defines the structures for inter-stage pipeline registers that hold instruction state as it flows through the pipeline.

### IF/ID Register

Holds state between Fetch and Decode stages.

struct IF_ID_Reg {
    Instruction instr;      // Fetched instruction
    int pc;                 // PC of this instruction
    bool predicted_taken;   // Branch prediction result
    bool valid;             // Is this register holding valid data?
};

### ID/EX Register

Holds state between Decode and Execute stages.

struct ID_EX_Reg {
    Instruction instr;      // Decoded instruction
    int pc;                 // PC of this instruction
    int rs1_val;            // Value read from rs1
    int rs2_val;            // Value read from rs2
    int imm;                // Sign-extended immediate
    int rd;                 // Destination register number
    bool predicted_taken;   // Propagated branch prediction
    bool valid;             // Is this register holding valid data?
};

### EX/MEM Register

Holds state between Execute and Memory stages.

struct EX_MEM_Reg {
    Instruction instr;      // Instruction in MEM stage
    int alu_result;         // Result from ALU
    int write_data;         // Data to write (for SW)
    int rd;                 // Destination register number
    bool valid;             // Is this register holding valid data?
};

### MEM/WB Register

Holds state between Memory and Writeback stages.

struct MEM_WB_Reg {
    Instruction instr;          // Instruction in WB stage
    int mem_data;               // Data loaded from memory
    std::vector<int> vec_data;  // Vector data for VLOAD
    int alu_result;             // ALU result (for non-memory ops)
    int rd;                     // Destination register number
    bool valid;                 // Is this register holding valid data?
};

# 4. ISA Module (src/isa/)

The ISA module defines the instruction set architecture including opcodes and instruction format.

## 4.1 Instruction.h

### Opcode Enumeration

Defines all supported opcodes including custom sparse extensions.

enum Opcode {
    // Base RISC-V Subset
    ADD,    // rd = rs1 + rs2
    SUB,    // rd = rs1 - rs2
    JMP,    // PC = PC + imm (unconditional jump)
    LW,     // rd = Mem[rs1 + imm]
    SW,     // Mem[rs1 + imm] = rs2
    BEQ,    // if (rs1 == rs2) PC += imm
    SLT,    // rd = (rs1 < rs2) ? 1 : 0
    ADDI,   // rd = rs1 + imm
    JR,     // PC = rs1 (return)
    
    // Custom Sparse Extensions
    LNZ,    // Load Non-Zero (skips zeros in hardware)
    ZMUL,   // Zero-Skip Multiply (1 cycle if operand is 0)
    ZMAC,   // Zero-Skip Multiply-Accumulate
    VLOAD,  // Vector Load (loads 4 words)
    BZERO,  // Branch if Zero
    MOV,    // Move (pseudo-instruction → ADDI)
    HALT    // Stop simulation
};

### Instruction Structure

struct Instruction {
    Opcode op;          // Operation code
    int rd;             // Destination register (0-31)
    int rs1;            // Source register 1 (0-31)
    int rs2;            // Source register 2 (0-31)
    int imm;            // Immediate value (signed)
    std::string raw_asm;// Original assembly text (for debugging)

    // Default constructor creates a NOP (HALT with rd=rs1=rs2=imm=0)
    Instruction() : op(HALT), rd(0), rs1(0), rs2(0), imm(0), raw_asm("NOP") {}
};

### Opcode Reference Table

| Opcode | Format | Description |
| --- | --- | --- |
| ADD | R-Type | rd = rs1 + rs2 |
| SUB | R-Type | rd = rs1 - rs2 |
| ADDI | I-Type | rd = rs1 + imm |
| SLT | R-Type | rd = (rs1 < rs2) ? 1 : 0 |
| LW | I-Type | rd = Mem[rs1 + imm] |
| SW | S-Type | Mem[rs1 + imm] = rs2 |
| BEQ | B-Type | if (rs1 == rs2) PC += imm |
| JMP | J-Type | PC = PC + imm |
| JR | I-Type | PC = rs1 |
| LNZ | I-Type | Load Non-Zero. Skip if zero. |
| ZMUL | R-Type | Zero-Skip Multiply. 1 cycle if 0. |
| VLOAD | I-Type | Vector Load. 4 words → vD. |
| BZERO | B-Type | Branch if rs1 == 0 |
| HALT | X-Type | Stop simulation |

## 4.2 ISA.h

Defines register ABI name mappings for debugging output.

const std::map<int, std::string> REG_ABI = {
    {0, "zero"},   // Hardwired zero
    {1, "ra"},     // Return address
    {2, "sp"},     // Stack pointer
    {5, "t0"},     // Temporary 0
    {6, "t1"},     // Temporary 1
    {7, "t2"},     // Temporary 2
    {10, "a0"},    // Argument 0
    {11, "a1"}     // Argument 1
};

# 5. Hardware Module (src/hardware/)

The hardware module contains models of processor hardware components including memory, registers, cache, and branch predictor.

## 5.1 Memory.h/.cpp

Models both instruction memory and data memory with cache simulation.

### Class Declaration

class Memory {
private:
    std::vector<int> data_mem;          // Data memory (4KB)
    std::vector<Instruction> instr_mem; // Instruction memory
    Cache cache;                        // L1 cache model

    const int HIT_LATENCY = 1;          // Cache hit: 1 cycle
    const int MISS_LATENCY = 10;        // Cache miss: 10 cycles

public:
    Memory();
    
    // Program loading
    void loadProgram(const std::vector<Instruction>& program);
    
    // Instruction fetch
    Instruction fetch(int pc);
    
    // Data access with latency
    int requestAccess(int addr);  // Returns latency in cycles
    int readData(int addr);
    void writeData(int addr, int value);

    // Cache statistics
    Cache& getCache();
    long long getCacheHits() const;
    long long getCacheMisses() const;
};

### Method Details

| Method | Description |
| --- | --- |
| Memory() | Constructor. Initializes 1024-word data memory to zero. |
| loadProgram(program) | Copies instruction vector to instruction memory. |
| fetch(pc) | Returns instruction at given PC. Returns NOP if out of bounds. |
| requestAccess(addr) | Queries cache. Returns HIT_LATENCY or MISS_LATENCY. |
| readData(addr) | Returns data at address. Returns 0 if out of bounds. |
| writeData(addr, value) | Writes value to address if within bounds. |

## 5.2 RegisterFile.h/.cpp

Implements the register file with 32 scalar registers (x0-x31) and 8 vector registers (v0-v7).

### Class Declaration

class RegisterFile {
private:
    int regs[32];       // 32 scalar registers (x0-x31)
    int vregs[8][4];    // 8 vector registers, 4 elements each

public:
    RegisterFile();
    
    // Scalar access
    int read(int reg_num);              // Read register (x0 always returns 0)
    void write(int reg_num, int value); // Write register (x0 writes ignored)
    
    // Vector access
    std::vector<int> readVector(int reg_num);
    void writeVector(int reg_num, const std::vector<int>& values);

    // Debug output
    void dump();  // Prints all non-zero registers
};

### Implementation Notes

x0 Hardwired to Zero: Reading x0 always returns 0. Writing to x0 is ignored (no-op).

Vector Registers: Each vector register holds 4 integers (128 bits total). Used by VLOAD instruction.

## 5.3 Cache.h/.cpp

Implements a direct-mapped cache with configurable size and block size.

### Class Declaration

class Cache {
private:
    struct CacheLine {
        bool valid;     // Line contains valid data
        int tag;        // Address tag
    };

    std::vector<CacheLine> lines;
    int size_bytes;     // Total cache size (default: 1024 bytes)
    int block_size;     // Block size (default: 64 bytes)
    int num_lines;      // Number of cache lines

    // Statistics
    long long hits;
    long long misses;

public:
    Cache(int size_bytes = 1024, int block_size = 64);

    // Access cache, returns true if hit, false if miss
    bool access(int addr);

    // Statistics getters
    long long getHits() const;
    long long getMisses() const;
};

### Cache Access Algorithm

bool Cache::access(int addr) {
    // Calculate index and tag
    int index = (addr / block_size) % num_lines;
    int tag = addr / (block_size * num_lines);
    
    // Check for hit
    if (lines[index].valid && lines[index].tag == tag) {
        hits++;
        return true;  // Cache hit
    } else {
        misses++;
        // Allocate line (write-allocate policy)
        lines[index].valid = true;
        lines[index].tag = tag;
        return false; // Cache miss
    }
}

## 5.4 BranchPredictor.h/.cpp

Implements a 1-bit dynamic branch predictor with direct-mapped table.

### Class Declaration

class BranchPredictor {
private:
    std::vector<bool> prediction_table;  // 1-bit per entry
    int size;                            // Table size (default: 1024)

public:
    BranchPredictor(int table_size = 1024);

    // Predict branch outcome (true = taken, false = not taken)
    bool predict(int pc);

    // Update predictor after branch resolution
    void update(int pc, bool actually_taken);
};

### Prediction Algorithm

bool BranchPredictor::predict(int pc) {
    int index = pc % size;
    return prediction_table[index];  // Return last outcome for this PC
}

void BranchPredictor::update(int pc, bool actually_taken) {
    int index = pc % size;
    prediction_table[index] = actually_taken;  // Update to actual outcome
}

# 6. Utilities Module (src/utils/)

## 6.1 Loader.h/.cpp

Parses assembly files and converts them to Instruction vectors.

### Class Declaration

class Loader {
private:
    std::map<std::string, int> label_map;  // Label → PC mapping
    
    // Helper methods
    int parseRegister(std::string reg_str);
    Opcode parseOpcode(std::string op_str);
    void parseOperands(Instruction& instr, const std::string& op_part, int current_pc);
    std::string trim(const std::string& str);

public:
    std::vector<Instruction> loadFromFile(const std::string& filename);
};

### Two-Pass Assembly

The loader uses a two-pass algorithm:

Pass 1: Scan for labels and record their PC addresses.

Pass 2: Parse instructions and resolve label references to offsets.

### Register Parsing

Supports both numeric (x0-x31) and ABI names:

| ABI Name | Register Number |
| --- | --- |
| Zero | 0 |
| Ra | 1 |
| Sp | 2 |
| t0 | 5 |
| t1 | 6 |
| t2 | 7 |
| s0 | 8 |
| s1 | 9 |
| a0 | 10 |
| a1 | 11 |
| t3 | 28 |
| t4 | 29 |
| t5 | 30 |
| t6 | 31 |

## 6.2 Stats.h/.cpp

Collects and reports performance statistics.

### Class Declaration

class Stats {
private:
    long long total_cycles;
    long long instructions_retired;
    long long stall_cycles;
    long long branch_flushes;
    long long zero_skips;
    
    // Instruction Mix
    long long alu_ops;
    long long memory_ops;
    long long branch_ops;
    
    // Cache Stats
    long long cache_hits;
    long long cache_misses;
    
    // Branch Prediction
    long long branch_predictions;
    long long branch_correct;

public:
    Stats();
    
    // Increment methods
    void addCycle();
    void addInstructionRetired();
    void addStall();
    void addBranchFlush();
    void addZeroSkip();
    void addAluOp();
    void addMemoryOp();
    void addBranchOp();
    
    // Setters
    void setCacheStats(long long hits, long long misses);
    void addBranchPrediction(bool correct);

    // Output
    void printReport();
};

### Performance Report Output

================================================================================
                       SPARSEFLOW PERFORMANCE REPORT                            
================================================================================
Total Cycles:                 349
Instructions Retired:         80
CPI (Cycles Per Instr):       4.36
IPC (Instr Per Cycle):        0.23

[Pipeline Events]
Stall Cycles (Hazards/Mem):   16 cycles lost
Branch Mispredict Flushes:    4 times
Zero-Skipping Savings:        5 ops skipped

[Instruction Mix]
ALU / Compute:                45
Memory (Load/Store):          25
Control Flow:                 14

[Cache Performance]
Cache Hits:                   28
Cache Misses:                 1
Cache Hit Rate:               96.6%

[Branch Prediction]
Predictions Made:             14
Correct Predictions:          10
Prediction Accuracy:          71.4%
================================================================================

## 6.3 TraceLogger.h/.cpp

Generates CSV trace files for pipeline visualization.

### Class Declaration

class TraceLogger {
private:
    std::ofstream file;
    bool enabled;

public:
    TraceLogger(const std::string& filename);
    ~TraceLogger();
    
    void logCycle(int cycle, 
                  const std::string& fetch_instr, 
                  const std::string& decode_instr, 
                  const std::string& execute_instr, 
                  const std::string& memory_instr, 
                  const std::string& writeback_instr,
                  const std::string& event);
};

### CSV Output Format

Cycle,IF,ID,EX,MEM,WB,Event
1,ADDI t0 x0 5,-,-,-,-,
2,ADD t1 t0 t0,ADDI t0 x0 5,-,-,-,
3,SW t1 0(t0),ADD t1 t0 t0,ADDI t0 x0 5,-,-,
4,LW t2 0(t0),SW t1 0(t0),ADD t1 t0 t0,ADDI,-,Forwarding
5,BEQ t1 t2 LOOP,LW t2 0(t0),SW t1 0(t0),ADD,ADDI,Stall (Load-Use)

# 7. Main Entry Point (main.cpp)

The entry point that initializes the simulator and runs the program.

int main(int argc, char* argv[]) {
    SparseFlowCore core;
    Loader loader;

    // Parse command-line arguments
    string filename = "test_program.asm";
    bool single_cycle = false;

    for (int i = 1; i < argc; ++i) {
        string arg = argv[i];
        if (arg == "--single-cycle") {
            single_cycle = true;
        } else {
            filename = arg;
        }
    }

    // Configure execution mode
    if (single_cycle) {
        cout << "Mode: Single-Cycle (Serialized Pipeline)" << endl;
        core.setSingleCycleMode(true);
    } else {
        cout << "Mode: Pipelined" << endl;
    }

    // Load and run program
    cout << "Loading program from: " << filename << endl;
    vector<Instruction> program = loader.loadFromFile(filename);

    if (program.empty()) {
        cout << "Error: Program is empty or file not found." << endl;
        return 1;
    }

    core.loadProgram(program);
    core.run();

    return 0;
}

### Command-Line Usage

# Run in pipelined mode (default)
./simulator program.asm

# Run in single-cycle mode
./simulator program.asm --single-cycle

# 8. Class Diagrams

## 8.1 Class Relationships

# 9. Data Flow

## 9.1 Pipeline Data Flow

## 9.2 Hazard Detection Flow

Load-Use Hazard Detection (in Decode stage):
═══════════════════════════════════════════════════════════════════

if (id_ex.instr.op == LW || id_ex.instr.op == LNZ) {
    if (id_ex.rd == if_id.instr.rs1 || id_ex.rd == if_id.instr.rs2) {
        // Insert bubble: stall IF/ID, insert NOP in ID/EX
        stall_pipeline = true;
    }
}

Control Hazard Detection (in Execute stage):
═══════════════════════════════════════════════════════════════════

if (branch_taken != predicted_taken) {
    // Misprediction: flush IF/ID, correct PC
    if_id.valid = false;
    pc = correct_target;
    branch_predictor.update(pc, branch_taken);
}

# 10. API Reference

## 10.1 SparseFlowCore Public API

| Method | Returns | Description |
| --- | --- | --- |
| SparseFlowCore() | void | Default constructor |
| loadProgram(prog) | void | Load instruction vector into memory |
| writeDataMemory(addr, val) | void | Pre-initialize data memory |
| run() | void | Execute simulation until HALT |
| setSingleCycleMode(enable) | void | Enable/disable single-cycle mode |

## 10.2 Memory Public API

| Method | Returns | Description |
| --- | --- | --- |
| loadProgram(program) | void | Load instruction vector |
| fetch(pc) | Instruction | Fetch instruction at PC |
| requestAccess(addr) | int | Get cache latency (1 or 10) |
| readData(addr) | int | Read from data memory |
| writeData(addr, value) | void | Write to data memory |
| getCacheHits() | long long | Get cache hit count |
| getCacheMisses() | long long | Get cache miss count |

## 10.3 Loader Public API

| Method | Returns | Description |
| --- | --- | --- |
| loadFromFile(filename) | vector<Instruction> | Parse assembly file and return instructions |

# Appendix: Build Instructions

## Requirements

C++17 compiler (GCC 7+, Clang 5+, MSVC 2017+)

CMake 3.10+ (optional)

## Manual Compilation (g++)

g++ -std=c++17 -O2 \
    -I src/core -I src/utils -I src/hardware -I src/isa \
    src/main.cpp \
    src/core/SparseFlowCore.cpp \
    src/utils/Loader.cpp \
    src/utils/Stats.cpp \
    src/utils/TraceLogger.cpp \
    src/hardware/Memory.cpp \
    src/hardware/RegisterFile.cpp \
    src/hardware/BranchPredictor.cpp \
    src/hardware/Cache.cpp \
    -o simulator

## CMake Build

mkdir build
cd build
cmake ..
make

## Running the Simulator

# Pipelined mode (default)
./simulator test_program.asm

# Single-cycle mode
./simulator test_program.asm --single-cycle

# With benchmark
./simulator benchmarks/sparse_90pct.asm